Sensor Connections – Fish Aquaculture Monitoring System

- Temperature Sensor

  - VCC → 3.3V

  - GND → GND

  - DATA → GPIO 4 (ESP32)

Notes: Use 10k pull up resistor for DATA pin.

- pH Sensor

  - VCC → 3.3V or 5V (module dependent)

  - GND → GND

  - AOUT → GPIO 34 (Analog Input)

- Turbidity Sensor - VCC → 5V

  - GND → GND

  - AOUT → GPIO 35

- TDS Sensor

  - VCC → 3.3V recommended

  - GND → GND

  - AOUT → GPIO 32

- MQ135 Air Quality Sensor

  - VCC → 5V

  - GND → GND

  - AOUT → GPIO 33

Safety Notes:

  - Always use common ground between ESP32 and sensors.

  - MQ135 requires 5V due to heater.

  - Turbidity readings may fluctuate, averaging recommended.

  - Ensure waterproofing for all water-contact sensors.